### Campus Safety - Safety Services

| SAFETY BULLETIN - PORTABLE SPACE HEATERS Winter is here and the temperatures are dropping. As a result, there is an increase in the use of portable space heaters. Unfortunately every year incidents of fires and damage occur from improper use of heaters. Recently an incident occurred involving a portable space heater in an office on another campus. The heater caught fire causing damage to the carpet and destroying the heater. If you’re going to use a space heater please remember the risk potential. Below are some basic safety do’s and don’ts for using space heaters… |  |
| --- | --- |
| 7 Things you should do… | 7 Things you shouldn’t do… |

| Contact Facilities - If your office temperature is uncomfortable, complete a Facilities Work Request so that the temperature concerns can be reviewed. 3 ft. Rule – keep your heater at least 3 ft. from combustible materials.. Inspect Cords – make sure the heater cord is not damaged or frayed.. Plug is Snug – make sure the heater is securely plugged into the outlet.. Certified – make sure your heater has a CSA© or UL© or other acceptable certification sticker.. Safety Features – choose a heater that has a thermostat and auto-shut off capabilities.. Turn it Off – ensure your heater is turned off when left unattended for extended periods of time.. | Crowd your Unit – don’t put anything around the front, sides or rear of your heater.. Extension Cords – don’t use extension cords or power bars; plug directly into the wall outlet.. Place on Tables – do not use your heater on top of a table or chair; use a stable, level surface.. Cords under Rugs – do not run the power cord underneath carpets, rugs or furniture.. Hot to Touch – do not use the heater if the power cord or receptacle feels hot to the touch.. Know your Size – do not use a heater that is designed for an area larger than you need. Contact Safety Services for questions/concerns and contact Facilities to request a work order.. |
| --- | --- |

| Campus Safety - Safety Services |  | Date of Issue: | 2019.10.07 |
| --- | --- | --- | --- |
|  |  | Revision #: |  |
|  |  | Revision Date: |  |